Protocol Demonstration Program (TERM)

TERM  merupakan satu program komunikasi yang sesuai untuk calling up bulletin board systems (BBS) dan sebagai satu program yang dapat menyalinkan file dari PC ke PC.  TERM  itself  is not part of the communications library, but rather it is provided as an example of a communications application using PPL4C.

TERM boleh menghantar satu standard Hayes standard AT command set string kepada modem. Satu initialization  string  dihantar oleh TERM diberikan pada AT_COMMAND_SET di mana yang diisytiharkan di dalam fail TERM.CFG.

#define AT_COMMAND_SET 1

TERM  juga menyokong  perkakasan flow control (RTS/CTS). Perkakasan flow control diperhatikan di mana constant RTS_CTS_CONTROL  diisytiharkan di dalam fail TERM.CFG.

#define RTS_CTS_CONTROL 1

TERM boleh dikonfigurasikan untuk melaksanakan fail skrip dengan setting

#define SCRIPTS 1

dalam TERM.CFG.

TERM boleh menukarkan fail dengan menggunakan ASCII XMODEM, YMODEM  dan  ZMODEM  protokol komunikasi.   TERM  dapat menerima wildcards di dalam name fail oleh itu multiple fail boleh dihantar dengan menggunakan YMODEM dan ZMODEM.  Pemasa protokol boleh dilaraskan dengan mengubahsuai constants SHORT_WAIT dan LONG_WAIT dalam fail PPL4C.C .

TERM juga boleh digunakan sebagai program penghantaran di antara PC dengan PC dengan menggunakan satu null modem  cable. Dalam kes ini, AT_COMMAND_SET dan RTS_CTS_CONTROL patut tidak diisytiharkan dalam fail TERM.CFG:

Pengguna dinasihatkan supaya jangan salin bertukar kable modem RTS  and  CTS,  di mana yang diperlukan bagi proses hardware  flow  control. Ini bermakna RTS_CTS_CONTROL tidak patuts diisytiharkan apabila menggunakan null modem cable kecuali pengguna yakin RTS dan CTS boleh salin bertukar guna.

Untuk melaksanakan TERM, taip TERM diikuti dengan port (1 to 4) dan baud rate (300, 600, 1200, 2400, 4800, 9600, 19200, 38400, 57600, or 115200). Sebagai contoh,  untuk memulakan TERM pada 9600 baud pada port COM2:

TERM 2 9600

XMODEM

Definasi perkataan ASCII berikut digunakan dalam XMODEM.

<SOH> = 01H : Digunakan untuk menunjukkan permulaan bingkai data.

<EOT> = 04H : Digunakan untuk menunjukkan tamat penghantaran bagi

sesuatu teks dan menutup perhubungan.

<ACK> = 06H : Digunakan oleh penerima untuk menjawab mesej yang

dihantar oleh penghantar.

<NAK> = 15H : Dihantar oleh penerima bahawa tidak bersedia.

<CAN> = 18H : Digunakan untuk membatalkan penghantaran.

XMODEM  adalah satu receiver  driven, asynchronous, 8 data bit protokol. Setiap packet adalah seperti berikut:

<SOH> <packet #> <compliment #> <data> <checksum>

di mana:

<SOH>         = 01H

<packet #>    = Packet number, mula pada 01, bertambah dengan 1,

dan disimpan dari 0FFH hingga 00H.

<compliment #>= 255 minus the packet number.

<data>        = 128 bytes of binary data.

<checksum>    = The sum of the data bytes. Bermula dengan sifar

ditambahkan setiap data byte kepada checksum.

Dengan menggunakan rightmost 8 bits.

Apabila penerima telah bersedia, ia akan menghantar  NAK setiap 10 saat sehingga NAK telah diberitahu oleh penghantar dengan menghantar packet pertama. Penghantar terus menghantar setiap packet mengikut giliran, sentiasa menunggu packet ACK sebelum menghantar data seterusnya.  Akhirnya,  bila penghantar telah selesai penghantaran , ia akan menghantar EOT menggantikan SOH untuk menyelesaikan proses penghantaran.

Program TERM.C

/*

**                    --- term.c ---

**

**  EXAMPLE CODE: Terminal emulator. Can transfer files using

**  ASCII, XMODEM, YMODEM, and ZMODEM protocols.

**

**  See TERM.CFG for configuration parameters.

**

**  Link with ALLOCSEG, TERM_IO, MODEM_IO, DIR_IO, CRC16, CRC32, DOS,

**  AMODEM, XYMODEM, XYPACKET, SI, OPCODES, and WIN_IO. Files SI.C and

**  OPCODES.C used for scripts only. See TERM makefiles and TERM.CFG.

**

**  Do NOT select YMODEM-G when using a null modem cable unless you are

**  certain that RTS & CTS are reversed ( this is usually not true ).

**

**  This example program (not the PCL4C library) is donated to

**  the Public Domain by MarshallSoft Computing, Inc. It is

**  provided as an example of the use of the PCL4C.

**

*/

#include <stdio.h>

#include <stdlib.h>

#include <ctype.h>

#include <fcntl.h>

#include <string.h>

#include <sys\types.h>

#include <sys\stat.h>

#include <malloc.h>

#include <dos.h>

#include <io.h>

#include <conio.h>

#include "term.cfg"

#include "pcl4c.h"

#include "ascii.h"

#include "term.h"

#include "dir_io.h"

#include "term_io.h"

#include "dos_io.h"

#include "modem_io.h"

#include "crc16.h"

#include "crc32.h"

#include "xymodem.h"

#include "zmodem.h"

#include "amodem.h"

#include "term.h"

#include "allocseg.h"

#include "timing.h"

#include "win_io.h"

#if SCRIPTS

#include "si.h"

#endif

#define DEBUG 0

#define ATTN_CHAR  0x1a

#define ABORT_CHAR CAN

#define FALSE 0

#define TRUE !FALSE

#define WORD unsigned int

#define NORMAL  0x07

#define INVERSE 0x70

#define MESSAGE_POS 48

/* local prototypes */

void ProcessESC(void);

void ShowStatus(void);

void StatusMsg(char,char *);

void CheckWin(int);

void SetASCII(void);

void SetX(void);

void SetY(void);

void SetG(void);

void SetZ(void);

/*** Global variables ***/

#define SIOBUF_SIZE 2048

#define BLKBUF_SIZE 1024

int  LastError = 0;     /* last reported error # */

int  Port;              /* current COM port [0..3] */

char FileSpec[15];      /* file name specification */

char FileName[15];      /* file name buffer */

char Buffer[BLKBUF_SIZE];      /* 1K block buffer */

char *BaudRate[10] =  {"300","600","1200","2400","4800","9600",

"19200","38400","57600","115200"};

int  BaudCode;          /* baud rate code ( index into BaudRate[] ) */

char *ModelText[4] = {"Small","Compact","Medium","Large"};

char NCGchar   = NAK;

int  OneKflag  = FALSE;

int  BatchFlag = FALSE;

char Protocol  = 'X';

char TermChar  = 0x00;

int  CharPace  = 0;

int  Timeout   = 10;

int  EchoFlag  = TRUE;

int  TraceFlag = FALSE;

int  Stream    = FALSE;

int MultiUART = 0;

int MultiIRQ = 0;

int MultiStatus = 0;

/*** main program ***/

void main(int argc,char *argv[])

{int DataFlag = FALSE;

char c;

int i, k;

int n, rc;

int Delta;         /* delta port status */

int Status;        /* port status */

int Version;

char Temp[81];

/* right number of parameters ? */

if((argc<3)||(argc>4))

{printf("Usage: 'TERM port baud' -- example 'TERM 1 9600 {script}'\n");

exit(1);

}

/* get port number from command line */

Port = atoi(argv[1]) - 1;

if((Port<0) || (Port>15))

{printf("Port must be 1 to 16\n");

exit(1);

}

/* get baud rate from command line */

BaudCode = BaudMatch(argv[2]);

if(BaudCode<0)

{printf("Cannot recognize baud rate = %s\n",argv[2]);

exit(1);

}

/* can ZMODEM stream ? */

if(BaudCode<=Baud19200) Stream = TRUE;

else Stream = FALSE;

#if 0

/*** Custom Configuration: 4 port card ***/

SioIRQ(COM1,IRQ4);

SioIRQ(COM2,IRQ3);

SioIRQ(COM3,IRQ4);

SioIRQ(COM4,IRQ3);

#endif

#if 0

/*** Custom Configuration: DigiBoard PC/8 ***/

MultiUART = 0x100; MultiIRQ = IRQ5; MultiStatus = 0x140;

printf("Configuring DigiBoard as COM1-COM8 (IRQ%d) @ 0x%x",

MultiIRQ,MultiUART);

SioDelay(18);

/* use 0x140 for odd IRQs & 0x141 for even IRQs */

SioPorts(8,COM1,MultiStatus,DIGIBOARD);

/* set DigiBoard UART addresses */

ErrorCheck( SioUART(Port,MultiUART+8*Port) );

/* set DigiBoard for IRQ */

ErrorCheck( SioIRQ(Port,MultiIRQ) );

#endif

#if 0

/*** Custom Configuration: BOCA 16 port dumbcard ***/

MultiUART = 0x280; MultiIRQ = IRQ11; MultiStatus = MultiUART + 7;

printf("Configuring BOCA Board as COM1-COM16 (IRQ%d) @ 0x%x",

MultiIRQ,MultiUART);

SioDelay(18);

/* use base port + 7 */

SioPorts(16,COM1,MultiStatus,BOCABOARD);

/* set BOCA Board UART addresses */

ErrorCheck( SioUART(Port,MultiUART+8*Port) );

/* set BOCA Board for IRQ */

ErrorCheck( SioIRQ(Port,MultiIRQ) );

#endif

/********************************************************/

/* setup receive buffer */

ErrorCheck( SioRxBuf(Port,AllocSeg(2048),Size2K) );

if(SioInfo('I'))

{/* setup transmit buffer */

ErrorCheck( SioTxBuf(Port,AllocSeg(2048),Size2K) );

}

/* set parms & reset (initialize) COM port */

SioParms(Port,NoParity,OneStopBit,WordLength8);

/* reset port */

rc = SioReset(Port,BaudCode);

if(rc<0) rc = SioReset(Port,BaudCode);

if(rc<0)

{printf("Cannot reset COM%d\n",1+Port);

exit(1);

}

/* set DTR and RTS */

SioDTR(Port,'S');

SioRTS(Port,'S');

/*

** !!! Define window screens !!!

** Use win_io function from now on

*/

/* define main screen window */

CheckWin( WinCreate(SCR_WIN,  0,0, 23,79) );

CheckWin( WinNormal(SCR_WIN) );

CheckWin( WinClear(SCR_WIN) );

/* define left status window */

CheckWin( WinCreate(LFT_WIN, 24,0, 24,30) );

CheckWin( WinInverse(LFT_WIN) );

CheckWin( WinClear(LFT_WIN) );

/* define right status window */

CheckWin( WinCreate(RGT_WIN, 24,32, 24,79) );

CheckWin( WinInverse(RGT_WIN) );

CheckWin( WinClear(RGT_WIN) );

/* display status message */

ShowStatus();

/* display some info */

WinPutString(SCR_WIN,"TERM: 11/11/95 [C/C++]\n\n");

Version = SioInfo('V');

sprintf(Temp,"      Library: %d.%d\n",Version>>4,0x0f&Version);

WinPutString(SCR_WIN,Temp);

sprintf(Temp," Memory Model: %s\n",ModelText[3&SioInfo('M')] );

WinPutString(SCR_WIN,Temp);

strcpy(Temp,"   Interrupts: ");

if(SioInfo('I')) strcat(Temp,"YES\n");

else strcat(Temp,"NO\n");

WinPutString(SCR_WIN,Temp);

WinPutString(SCR_WIN,"   Parameters: 8N1\n");

#if RTS_CTS_CONTROL

SioFlow(Port,10*ONE_SECOND);

WinPutString(SCR_WIN," Flow Control: YES\n");

#else

WinPutString(SCR_WIN," Flow Control: NO\n");

#endif

/* Set FIFO level */

WinPutString(SCR_WIN,"   16550 UART: ");

if( SioFIFO(Port,LEVEL_14) ) WinPutString(SCR_WIN,"YES\n");

else WinPutString(SCR_WIN,"NO\n");

/* clear PCL4C receive buffer */

ErrorCheck( SioRxClear(Port) );

WinPutChar(SCR_WIN,'\n');

/* wait for other end to say its ready */

WinPutString(SCR_WIN,"  <<Waiting for DSR>>\n");

while( !SioDSR(Port) )

{if(kbhit()||SioBrkKey()) MyExit(0,"Aborted by user");

WinPutChar(SCR_WIN,'.');

SioDelay(18);

}

WinPutString(SCR_WIN,"  <<DSR on>>\n");

#if RTS_CTS_CONTROL

WinPutString(SCR_WIN,"  <<Waiting for CTS>>\n");

while( !SioCTS(Port) )

{if(kbhit()||SioBrkKey()) MyExit(0,"Aborted by user");

WinPutChar(SCR_WIN,'.');

SioDelay(18);

}

WinPutString(SCR_WIN,"  <<CTS on>>\n");

#endif

#if AT_COMMAND_SET

/* initialize (Hayes compatible) modem */

ModemSendTo(Port,4,"AT!");

ModemEcho(Port,10);

ModemSendTo(Port,4,"AT E1 S7=60 S11=60 V1 X1 Q0!");

if(ModemWaitFor(Port,5*ONE_SECOND,TRUE,"OK"))

{/* modem is ready */

WinPutString(SCR_WIN,"\n  <<MODEM READY>>");

}

else WinPutString(SCR_WIN,"\n  <<WARNING: Expected OK not received>>\n");

#endif

SetX();

#if SCRIPTS

if(argc==4)

{/* call script interpreter */

sprintf(Temp,"Starting script %s\n",argv[3]);

WinPutString(SCR_WIN,Temp);

i = si(Port,argv[3],FALSE);

if(i<0) SaySiErr(i);

}

#endif

/* enter terminal loop */

SioRxClear(Port);

WinPutString(SCR_WIN,"  <<Entering terminal loop>>\n");

while(1)

{/* Control-BREAK ? */

if(SioBrkKey()) MyExit(0,"User pressed Ctrl-BREAK");

/* keep TX rolling */

SioTxFlush(Port);

/* was key pressed ? */

if(kbhit())

{/* read key press */

i = getch();

/* ^B sends BREAK */

if((char)i=='\02')

{/* send break */

WriteMsg("Sending BREAK");

SioBrkSig(Port,'A');

SioDelay(10);

SioBrkSig(Port,'C');

}

else if((char)i==ATTN_CHAR)

{/* process user's request */

ProcessESC();

ShowStatus();

continue;

}

else CharPut(Port,(char)i);

}

/* was break detected ? */

if( SioBrkSig(Port,'D') ) WriteMsg("BREAK detected ");

/* check DSR */

if(TestDSR(Port))

{/* any incoming over serial port ? */

i = CharGet(Port,1);

if(i>-1)

{/* good character */

if(DataFlag&((i<0x20)||(i>0x7e)))

{WinPutChar(SCR_WIN,'^');

WinPutChar(SCR_WIN,(char)('@'+i));

}

else WinPutChar(SCR_WIN,(char)i);

}

}

} /* end -- key pressed */

}

/*** find baud rate string in table ***/

int BaudMatch(char *ptr)

{int i;

/* find baud rate in table */

for(i=0;i<10;i++) if(strcmp(BaudRate[i],ptr)==0) return(i);

return(-1);

}

/*** user pressed Escape */

void ProcessESC(void)

{int i;

int rc;

int c1, c2;

char Answer[2]; /* answer array */

/* user pressed ATTN_CHAR */

Answer[0] = '?';

Answer[1] = '\0';

WriteMsg("Q)uit P)rotocol S)end R)eceive T)race: ");

ReadMsg(Answer,1,39);

if(strlen(Answer)) switch(toupper(Answer[0]))

{

case 'T':

/*SetIOtrace(TRUE);*/

TraceFlag = !TraceFlag;

WriteIntMsg("Trace = ",1&TraceFlag);

MsgEcho(TraceFlag);

break;

case 'P':

WriteMsg("A)scii X)modem Y)modem Z)modem: ");

ReadMsg(Answer,1,32);

if(strlen(Answer)) switch( toupper(Answer[0]) )

{case 'A':

SetASCII();

break;

case 'X':

SetX();

break;

case 'Y':

SetY();

break;

case 'Z':

SetZ();

break;

#if RTS_CTS_CONTROL

case 'G':

SetG();

break;

#endif

default:

WriteMsg("Must answer X, Y, or Z");

break;

}

ShowStatus();

break;

case 'Q':

SetIOtrace(FALSE);

MyExit(0,"User QUIT");

break;

case 'R':

StatusMsg(ABORT_CHAR,"aborts");

*FileName = '\0';

switch(Protocol)

{case 'A':

/* Ascii */

RxAscii(Port,FileName,Buffer,BLKBUF_SIZE,SIOBUF_SIZE,FALSE,TermChar,Timeout,EchoFlag);

break;

case 'X':

XmodemRx(Port,FileName,Buffer,NCGchar);

break;

case 'Y':

YmodemRx(Port,Buffer,NCGchar);

break;

case 'Z':

ZmodemRx(Port,Stream);

break;

}

while(kbhit()) getch();

break;

case 'S':

*FileName = '\0';

StatusMsg(ABORT_CHAR,"aborts");

switch(Protocol)

{case 'A': /* Ascii */

TxAscii(Port,FileName,Buffer,BLKBUF_SIZE,FALSE,CharPace,TermChar,EchoFlag);

break;

case 'X':

XmodemTx(Port,FileName,Buffer,OneKflag);

break;

case 'Y':

YmodemTx(Port,FileName,Buffer);

break;

case 'Z':

ZmodemTx(Port,FileName,Stream);

break;

}

while(kbhit()) getch();

break;

default:

WriteHexMsg("Must answer Q, P, S, R, or T, not ",*Answer);

break;

} /* end switch */

}

/*** show menu message ***/

void StatusMsg(char Attn,char *Msg)

{char Temp[50];

sprintf(Temp," COM%d %s %c [ ^%c %s ]",

1+Port,BaudRate[BaudCode],Protocol,Attn+'@',Msg);

WinClear(LFT_WIN);

WinPutString(LFT_WIN,Temp);

WinGetPos(SCR_WIN,NULL,NULL);

}

void ShowStatus(void)

{

StatusMsg(ATTN_CHAR,"for Menu");

}

/*** exit program ***/

void MyExit(int code, char *msg)

{int i, rc;

char Temp[80];

if(code<0)

{WinPutChar(SCR_WIN,' ');

SioError(code);

WinPutChar(SCR_WIN,'\n');

}

SioDone(Port);

sprintf(Temp,"\rTERMINATING: %s\n",msg);

WinPutString(SCR_WIN,Temp);

SetCursor(6,7);

exit(0);

}

/*** check for error ***/

void ErrorCheck(int Code)

{/* trap PCL error codes */

if(Code<0)

{

WinPutChar(SCR_WIN,' ');

SioError(Code);

SioDone(Port);

exit(1);

}

} /* end ErrorCheck */

int GetBufSize(void)

{int BufSize = SIOBUF_SIZE;

return BufSize;

}

void CheckWin(int n)

{if(n<0)

{printf("\nWin error %d\n\n",n);

exit(1);

}

}

/* set protocol parameters */

void SetASCII(void)

{Protocol = 'A';

TermChar = 0x18;  /* CAN or control-X */

CharPace = 0;     /* no inter-byte delay */

Timeout = 7;      /* timeout after 7 seconds */

EchoFlag = TRUE;  /* do local echo */

WriteMsg("Protocol = ASCII");

}

void SetX(void)

{Protocol = 'X';

OneKflag = FALSE;

BatchFlag = FALSE;

NCGchar = NAK;

WriteMsg("Protocol = XMODEM");

}

void SetY(void)

{Protocol = 'Y';

OneKflag = TRUE;

BatchFlag = TRUE;

NCGchar = 'C';

WriteMsg("Protocol = YMODEM");

}

void SetG()

{Protocol = 'G';

OneKflag = TRUE;

BatchFlag = TRUE;

NCGchar = 'G';

WriteMsg("Protocol = YMODEM-G");

}

void SetZ(void)

{Protocol = 'Z';

WriteMsg("Protocol = ZMODEM");

}

Program AMODEM.C

/*

**  ASCII text file transfer using XON / OFF flow control protocol.

**  Transfer ASCII files only. Do not attempt to transfer binary files.

*/

#include <stdio.h>

#include <stdlib.h>

#include <fcntl.h>

#include <string.h>

#include <io.h>

#include <conio.h>

#include <sys\types.h>

#include <sys\stat.h>

#include "pcl4c.h"

#include "ascii.h"

#include "term_io.h"

#include "xymodem.h"

#include "amodem.h"

#include "term.h"

#include "win_io.h"

#define FALSE 0

#define TRUE !FALSE

#define ONE_SECOND 18

char lastXchar;     /* last XON or XOFF sent */

int TxAscii(

int Port,            /* COM port [0..3] */

char Filename[],     /* filename buffer */

char Buffer[],       /* data buffer */

int Length,          /* size of Buffer */

int SyncFlag,        /* synchronize with XON 1st */

int CharPace,        /* millisecond delay after sending each character */

int TermChar,        /* termination character (0x00 ==> none) */

int EchoFlag)        /* do local echo if TRUE */

{int i;

int Code;           /* return code */

int Handle;         /* file Handle */

char LastChar;      /* last character sent */

int TxChars = 0;    /* # characters transmitted */

int Count;          /* # bytes read from disk */

char Temp[81];      /* temporary buffer */

/* begin */

if(!FetchName(Filename)) return FALSE;

Handle = open(Filename,O_RDONLY|O_BINARY,S_IREAD);

if(Handle<0)

{strcpy(Temp,"Cannot open ");

strcat(Temp,Filename);

WriteMsg(Temp);

return FALSE;

}

/* do we wait for XON before starting ? */

if(SyncFlag)

{/* wait for incoming XON */

WriteMsg("ASCII: Waiting for XON");

while(1)

{if(SioBrkKey())

{WriteMsg("Canceled by USER");

return FALSE;

}

Code = CharGet(Port,ONE_SECOND);

if(Code==-1) continue;

if(Code<0) return FALSE;

if((char)Code==XON)

{WriteMsg("ASCII: initial XON received");

/* slight delay */

SioDelay(ONE_SECOND/2);

break;

}

/* received character not XON */

WinPutChar(SCR_WIN,(char)Code);

}

}

/* begin transfer */

lastXchar =  XON;

WriteMsg("ASCII: Starting send");

while(kbhit()) getch();

/* clear comm port */

SioRxClear(Port);

/* send ascii file ( stop at ^Z ) */

while(1)

{/* read next buffer from disk */

Count = read(Handle,Buffer,Length);

if(Count==0) break;

if(Count<0)

{SayError(Port,"Error on disk read");

return FALSE;

}

/* send one byte at a time */

for(i=0;i<Count;i++)

{/* User ABORTS ? */

if(UserAborts(Port)) return FALSE;

/* send byte */

LastChar = Buffer[i];

if(EchoFlag) WinPutChar(SCR_WIN,LastChar);

/* send the character */

CharPut(Port,LastChar);

if(CharPace>0) SioDelay(CharPace);

/* slight delay after each line */

if(LastChar==LF) SioDelay(4*(CharPace+1));

TxChars++;

/* ^Z marks the end of a text file */

if(LastChar==CTLZ) break;

/* check for incoming XON */

if(lastXchar==XON)

{/* check for incoming XON / XOFF */

Code = CharGet(Port,0);

if(Code>0)

{/* is byte a XOFF ? */

if((char)Code==XOFF)

{/* wait for XON */

WriteMsg("XOFF received");

lastXchar = XOFF;

/* wait for XON */

while(1)

{/* user want to quit ? */

if(SioBrkKey())

{WriteMsg("Canceled by USER");

return FALSE;

}

Code = CharGet(Port,ONE_SECOND);

if((char)Code==XON)

{WriteMsg("XON  received");

lastXchar = XON;

break;

}

} /* end -- while */

} /* end -- if(XOFF) */

} /* end -- if(Code) */

} /* end -- if(XON) */

} /* end -- for(i) */

} /* end -- while */

close(Handle);

/* send termination character */

if(TermChar) CharPut(Port,(char)TermChar);

sprintf(Temp,"ASCII: %d chars sent.",TxChars);

WriteMsg(Temp);

return TRUE;

} /* end -- TxAcsii */

int RxAscii(

int Port,            /* COM port [0..3] */

char Filename[],     /* filename buffer */

char Buffer[],       /* data buffer */

int Length,          /* length of data buffer */

int RxQueSize,       /* size of PCL receive buffer */

int SyncFlag,        /* synchronize with XON 1st */

int TermChar,        /* termination character (0x00 ==> none) */

int TimeOut,         /* delay (seconds) before assuming that sender is done */

int EchoFlag)        /* do local echo if TRUE */

{int i;

int Handle;         /* file Handle */

int Code;           /* return code */

int lo;             /* receive queue low water mark */

int hi;             /* receive queue high water mark */

int Index;          /* buffer index */

int QueSize;        /* current PCL receive queue size */

int RxChars = 0;    /* # received chars */

int Count;          /* # characters written to disk */

char Temp[81];      /* temporary buffer */

long LastTime;      /* time last character was received */

/* begin */

lastXchar = XON;

lo = RxQueSize / 8;

hi = 5 * lo;

WriteMsg("ASCII: Starting receive ");

while(kbhit()) getch();

/* clear comm port */

/*SioRxClear(Port);*/

/* open file passed in Filename[] for write */

if(!FetchName(Filename)) return FALSE;

Handle = open(Filename,O_CREAT|O_TRUNC|O_WRONLY|O_BINARY,S_IWRITE);

if(Handle<0)

{strcpy(Temp,"Cannot open ");

strcat(Temp,Filename);

WriteMsg(Temp);

return FALSE;

}

/* sync with XON 1st ? */

if(SyncFlag)

{WriteMsg("ASCII: Sending initial XON");

while(1)

{CharPut(Port,XON);

Code = CharGet(Port,ONE_SECOND);

if(Code==-1) continue;

/* transmitter is sending ! */

SioUnGetc(Port,(char)Code);

break;

}

}

/* receive text */

LastTime = SioTimer();

Index = 0;

while(1)

{/* user want to quit ? */

if(SioBrkKey())

{WriteMsg("Canceled by USER");

return FALSE;

}

/* check on PCL receive queue size */

QueSize = SioRxQue(Port);

if((QueSize>hi)&&(lastXchar==XON))

{CharPut(Port,XOFF);

lastXchar = XOFF;

WriteMsg("sending XOFF(1)");

/*sprintf(Temp,"\nQueSize=%d lo=%d hi=%d\n",QueSize,lo,hi);*/

/*WriteMsg(Temp);*/

}

if((QueSize<lo)&&(lastXchar==XOFF))

{CharPut(Port,XON);

lastXchar = XON;

WriteMsg("sending XON ");

}

/* User ABORTS ? */

if(UserAborts(Port)) return FALSE;

/* get next byte */

Code = CharGet(Port,ONE_SECOND);

if(Code==-1)

{/* done if have exceeded timeout */

if(SioTimer()-LastTime>ONE_SECOND*TimeOut)

{/* sender must be done */

Buffer[Index] = CTLZ;

break;

}

continue;

}

/* ignore 1st character if it is a 0 */

if((RxChars==0)&&((char)Code=='\0')) continue;

LastTime = SioTimer();

/* ignore XON & XOFF ( since we are the receiver ) */

if((char)Code==XON) continue;

if((char)Code==XOFF) continue;

/* received a character */

Buffer[Index++] = (char)Code;

RxChars++;

/* TermChar marks the end of a text file */

if((char)Code==TermChar)

{/* replace TermChar with ^Z */

Buffer[Index-1] = CTLZ;

RxChars++;

break;

}

if(EchoFlag) WinPutChar(SCR_WIN,(char)Code);

if(Index==Length)

{/* send XOFF to transmitter */

CharPut(Port,XOFF);

lastXchar = XOFF;

WriteMsg("sending XOFF(2)");

/* write disk file */

Count = write(Handle,Buffer,Index);

if(Count<0)

{SayError(Port,"Disk write error");

SioDelay(ONE_SECOND);

return FALSE;

}

/* send XON to sender */

CharPut(Port,XON);

lastXchar = XON;

WriteMsg("sending XON ");

Index = 0;

} /* end -- if */

} /* end -- while */

/* write any remaining data in buffer */

if(Index>0)

{Count = write(Handle,Buffer,Index);

if(Count<0)

{SayError(Port,"Disk write error");

SioDelay(ONE_SECOND);

return FALSE;

}

} /* end -- if */

close(Handle);

sprintf(Temp,"ASCII: %d chars received.",RxChars);

WriteMsg(Temp);

return TRUE;

} /* RxAscii */

int UserAborts(int Port)

{char UserChar;

/* user aborts ? */

if(kbhit())

{UserChar = (char)getch();

if(UserChar==CAN)

{TxCAN(Port);

CharPut(Port,ETX);

WriteMsg("*** Canceled by USER ***");

return TRUE;

}

/* send user char */

CharPut(Port,UserChar);

}

return FALSE;

} /* UserAborts */